|  | ## ΑΙΤΗΣΗ ΓΙΑ ΠΙΣΤΟΠΟΙΗΣΗ non ETOPS (180 min) Application Form for non ETOPS (180 min) Approval (Airworthiness & Operational Approval Conformance Document) |
| --- | --- |

| REFERENCES | ###### ISSUE DATE | TITLE |
| --- | --- | --- |
| 965/2012 CAT.OP.MPA.140 | 10/2012 | Technical requirements and administrative procedures related to Air Operations |

| ##### OPERATOR : |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft MFG | GULFSTREAM | Aircraft Type | GVI (G650ER) | Aircraft Series |  |
| Registration Marks | MFG Serial No |  |  |  |  |
| SX-GJA | 6306 |  |  |  |  |

# Introduction

As prescribed in 965/2012 -CAT.OP.MPA.140 (a)(2)-, an operator may not operate a twin turbo-jet powered airplane having a maximum approved passenger seating configuration of 19 or less and a MTOM less than 45.360Kg beyond 120 minutes from an adequate aerodrome at the one engine inoperative cruise speed calculated in accordance with CAT.OP.MPA.140 (b) unless approved by the HCAA.

This 120 minute threshold may be exceeded by no more than 60 minutes. In order for operations between 120 and 180 minutes to be approved, due account should be taken of the aeroplane’s design and capabilities (as outlined below) and an operator’s experience related to such operations.

Operator  successfully operates under such operation since  19-12-2008 and has accumulate sufficient experience in order to obtain an approval to exceed the normal threshold of 120 min to 180 min for Type of Aircraft G650ER (SX-GJA, MSN 6306).

This compliance document based on:

- 965/2012 - CAT.OP.MPA.140 (a)(2),

- 965/2012 - AMC1 CAT.OP.MPA.140

# Systems capability

Airplanes should be certificated to CS-25 as appropriate or equivalent (e.g. FAR-25). With respect to the capability of the airplane systems, the objective is that the airplane is capable of a safe diversion from the maximum diversion distance with particular emphasis on operations with one engine inoperative or with degraded system capability. To this end, the operator should give consideration to the capability of the following systems to support such a diversion:

## Propulsion systems

The airplane engine should meet the applicable specifications prescribed in CS-25 and CS-E or equivalent (e.g. FAR-25, FAR-E), concerning engine TC, installation and system operation. In addition to the performance standards established by the Agency or competent authority at the time of engine certification, the engines should comply with all subsequent mandatory safety standards specified by the Agency or competent authority, including those necessary to maintain an acceptable level of reliability. In addition, consideration should be given to the effects of extended duration single-engine operation (e.g. the effects of higher power demands such as bleed and electrical).

## Airframe systems

With respect to electrical power, three or more reliable as defined by CS-25 or equivalent (e.g. FAR-25) and independent electrical power sources should be available, each of which should be capable of providing power for all essential services which should at least include the following:

(i) sufficient instruments for the flight crew providing, as a minimum, attitude, heading, airspeed and altitude information;

(ii) appropriate pitot heating;

(iii) adequate navigation capability;

(iv) adequate radio communication and intercommunication capability;

(v) adequate flight deck and instrument lighting and emergency lighting;

(vi) adequate flight controls;

(vii) adequate engine controls and restart capability with critical type fuel (from the stand-point of flame-out and restart capability) and with the airplane initially at the maximum relight altitude;

(viii) adequate engine instrumentation;

(ix) adequate fuel supply system capability including such fuel boost and fuel transfer functions that may be necessary for extended duration single or dual-engine operation;

(x) such warnings, cautions and indications as are required for continued safe flight and landing;

(xi) fire protection (engines and auxiliary power unit (APU));

(xii) adequate ice protection including windshield de-icing; and

(xiii) adequate control of the flight crew compartment and cabin environment including heating and pressurization.

The equipment including avionics necessary for extended diversion times should have the ability to operate acceptably following failures in the cooling system or electrical power systems.

For single-engine operations, the remaining power electrical, hydraulic, and pneumatic should continue to be available at levels necessary to permit continued safe flight and landing, and to provide those services necessary for the overall safety of the passengers and crew. As a minimum, following the failure of any two of the three electrical power sources, the remaining source should be capable of providing power for all of the items necessary for the duration of any diversion. If one or more of the required electrical power sources are provided by an APU, hydraulic system or air driven generator/ram air turbine (ADG/RAT), the following criteria should apply as appropriate:

- to ensure hydraulic power (hydraulic motor generator) reliability, it may be necessary to provide two or more independent energy sources;

- the ADG/RAT, if fitted, should not require engine dependent power for deployment; and

- The APU should meet the criteria in 2.3 below paragraph.

## APU

The APU if required for extended range operations, should be certified as an essential APU and should meet the applicable CS-25 and CS-APU provisions or equivalent (e.g. FAR-25).

2.4     Fuel supply system

Consideration should include the capability of the fuel supply system to provide sufficient fuel for the entire diversion taking account of aspects such as fuel boost and fuel transfer.

Engine Events and corrective action.

- All engine events and operating hours should be reported by the operator to the airframe and engine supplemental type certificate (STC) holders as well as to the HCAA.

- These events should be evaluated by the operator in consultation with the HCAA and with the engine and airframe (S)TC holders. The competent authority may consult the Agency to ensure that world wide data are evaluated.

- Where statistical assessment alone is not applicable, e.g. where the fleet size or accumulated flight hours are small, individual engine events should be reviewed on a case-by-case basis.

- The evaluation or statistical assessment, when available, may result in corrective action or the application of operational restrictions.

- Engine events could include engine shutdowns, both on ground and in-flight, excluding normal training events, including flameout, occurrences where the intended thrust level was not achieved or where crew action was taken to reduce thrust below the normal level for whatever reason, and unscheduled removals.

- Arrangements to ensure that all corrective actions required by the Agency are implemented.

# Maintenance:

The maintenance program in accordance with Annex I to Regulation (EU) No 1321/2014 (Part-M) should be based upon reliability program including, but not limited to, the following elements:

  - Engine oil consumption programs: such programs are intended to support engine condition trend monitoring; and

  - Engine condition monitoring program: a program for each engine that monitors engine performance parameters and trends of degradation that provides for maintenance actions to be undertaken prior to significant performance loss or mechanical failure.

# Flight Crew Training:

Flight crew training for this type of operation should include, in addition to the requirements of Subpart FC of Annex III (ORO.FC), particular emphasis on the following:

## Fuel management

Verifying required fuel on board prior to departure and monitoring fuel on board en-route including calculation of fuel remaining. Procedures should provide for an independent cross-check of fuel quantity indicators, e.g. fuel flow used to calculate fuel burned compared to indicate fuel remaining. Confirmation that the fuel remaining is sufficient to satisfy the critical fuel reserves.

## One-engine inoperative performance data

Procedures for single and multiple failures in-flight that may give rise to go/no-go and diversion decisions – policy and guidelines to aid the flight crew in the diversion decision making process and the need for constant awareness of the closest weather-permissible alternate aerodrome in terms of time. One Engine Inoperative (OEI) performance data: drift down procedures and OEI service ceiling data.

## Weather reports and flight requirements

Weather reports and flight requirements: meteorological aerodrome reports (METARs) and aerodrome forecast (TAF) reports and obtaining in-flight weather updates on the en-route alternate (ERA), destination and destination alternate aerodromes. Consideration should also be given to

forecast winds including the accuracy of the forecast compared to actual wind experienced during flight and meteorological conditions along the expected flight path at the OEI cruising altitude and throughout the approach and landing.

## Pre-departure check

A pre-departure check, additional to the pre-flight inspection required by Part-M should be reflected in the operations manual. Flight crew members who are responsible for the pre-departure check of an airplane should be fully trained and competent to do it. The training program required should cover all relevant tasks with particular emphasis on checking required fluid levels.

# MEL

The MEL should take into account all items specified by the manufacturer relevant to operations in accordance with this AMC.

# Dispatch/Flight Planning Requirements:

The operator’s dispatch requirements should address the following:

## Fuel and oil supply

An airplane should not be dispatched on an extended range flight unless it carries sufficient fuel and oil to comply with the applicable operational requirements and any additional reserves determined in accordance with sub-paragraphs (a)(i) (ii) and (iii) below.

(i) Critical fuel scenario

The critical point is the furthest point from an alternate aerodrome assuming a simultaneous failure of an engine and the pressurization system. For those airplanes that are type certificated to operate above Flight Level 450, the critical point is the furthest point from an alternate aerodrome assuming an engine failure. The operator should carry additional fuel for the worst case fuel burn condition (one engine vs two engines operating), if this is greater than the additional fuel calculated in accordance with CAT.OP.MPA, as follows:

A. Fly from the critical point to an alternate aerodrome:

- At 10 000ft; or

- At 25 000ft or the single-engine ceiling, whichever is lower, provided that all occupants can be supplied with and use supplemental oxygen for the time required to fly from the critical point to an alternate aerodrome; or

- At the single-engine ceiling, provided that the airplane is type certificated to operate above Flight Level 450.

B. Descend and hold at 1 500 feet for 15 minutes in ISA conditions;

C. Descend to the applicable MDA/DH followed by a missed approach (taking into account the complete missed approach procedure); followed by

D. A normal approach and landing.

(ii) Ice protection

Additional fuel used when operating in icing conditions (e.g. operation of ice protection systems (engine/airframe as applicable)) and, when manufacturer’s data is available, take account of ice accumulation on unprotected surfaces if icing conditions are likely to be encountered during

a diversion;

(iii) APU operation

If an APU has to be used to provide additional electrical power, consideration should be given to the additional fuel required.

## Communication facilities

The availability of communications facilities in order to allow reliable two-way voice communications between the aeroplane and the appropriate ATC unit at OEI cruise altitudes.

## Aircraft Technical Log

Review to ensure proper MEL procedures, deferred items, and required maintenance checks completed.

## En-route alternate aerodrome(s)

Ensuring that ERA aerodromes are available for the intended route, within the distance flown in 180 minutes based upon the OEI cruising speed which is a speed within the certificated limits of the aeroplane, selected by the operator and approved by the competent authority, confirming that, based on the available meteorological information, the weather conditions at ERA aerodromes are at or above the applicable minima for the period of time during which the aerodrome(s) may be used.

Table 1: Planning minima

| Approach facility | Alternate aerodrome ceiling | Weather minima RVR/VIS |
| --- | --- | --- |
| PA | DA/H +200 ft | RVR/VIS +800 m |
| NPA | MDA/H +400 ft | RVR/VIS +1 500 m |

8. GENERAL APPLICANT INFORMATION

| General Information |  |  |  |
| --- | --- | --- | --- |
| 8. 1 | Applicant: GAINJET AVIATION SA |  |  |
|  | Contact person (Name): Jason Aivaliotis |  |  |
|  | Tel: 210 9636101 (230) |  |  |
|  | Fax/e-mail: jason@gainjet.com |  |  |
|  | Aeroplane Registration: SX-GJA |  |  |
|  | Aeroplane Manufacturer: GULFSTREAM |  |  |
|  | Aeroplane Type Designation: GVI (G650ER) |  |  |
|  | Aeroplane Serial No.: 6306 |  |  |
|  | MAPSC: 16 |  |  |
|  | MTOM: 46992 |  |  |
|  | Engine Manufacturer: Rolls Royce |  |  |
|  | Engine Type Designation: BR700-725A1-12 |  |  |
|  | APU Manufacturer: Honeywell |  |  |
|  | APU Type Designation: RE 220 (GVI) |  |  |
|  | Total operator's airframe fleet hours: 9986.6 |  |  |
|  | Total operator's airframe fleet cycles : 3938.0 |  |  |
|  | Total operator' engine hours: 9986.6 |  |  |
| 8.2 | Area of operation as defined in the AOC : N70.00.00W180.00.00, N70.00.00 El 80.00.00, S60.00.00 W 180.00.00. S60.00.00 E 180.00.00 (Note: Operator has to check the Area of Operation as defined in the AOC and the Area of operation for non-ETOPS .) Operator response Refer to Attachment 8.2: GR-024/OS-17 29-07-20 |  |  |
| 8.3 | 1) AIRCRAFT EASA TCDS Number : TCDS No.: EASA.IM.A.169 Issue 6 Operator response Refer to Attachment 8.3.1: 8.3.1 - TCDS _EASA.IM_.A.169 Iss 6 |  |  |
|  | 2) ENGINE EASA TCDS Number : TCDS No.E.018 Issue 14 Operator response Refer to Attachment 8.3.2: 8.3.2 - TCDS E 018 issue 14_05.12.19 |  |  |
|  | 3) APU Manufacter/Type : Honeywell International Inc / RE220 [GVI] Operator response Refer to Attachment 8.3.3: 8.3.3 - 6306 APU Logbook |  |  |
|  | (Note 1 : In case that EASA TCDS not available refer to FAA TCDS or as applicable) (Note 2: TCDS should be attached to this conformance report) |  |  |
| 8.4 | 1) Operator Fleet size: Aeroplane types operated & Registration Number(s): B757-200 (SX-RFA), EMB-135BJ-LEGACY 600 (SX-DGM), GIV-G450 (SX-GAB) 2012 till 2019. (Note: Operator has to submit a copy of the latest current AOC approval ) Operator response Refer to Attachment 8.2 : GR-024/OS-17, 29-07-20 |  |  |
|  | 2) Number of month’s/years of operational experience with specific engine/airframe combination: Operator response : 2 Years |  |  |
|  | 3) Number of domestic sectors: Operator response: 02 |  |  |
|  | 4) Number of long range sectors: Operator response : 23 |  |  |
|  | 5) Unscheduled engine removal rate (URR) for the operator fleet (URR rate per 1000 engine flight hours): Operator response : Null , 8.7.2 - SX-GJA Reliability data 100820 |  |  |
|  | 6) In-flight shutdown (IFSD) rate (all causes) including the 12-month rolling average for the operator fleet (IFSD per 1000 engine flight hours) Operator response : Null, 8.7.2 - SX-GJA Reliability data 100820 |  |  |
| 8.5 | The requested aircraft for non-ETOPS 180 has additionally the following approvals : RVSM YES B-RNAV □ RNP-10 YES MNPS YES AWO P-RNAV YES Other RNP-APRCH, RNAV-1 Operator response Refer to Attachment 8.5.1/2/3/4/5: 8.5.1 - 1485 RVSM SX GJA_24883, 8.5.2 - 1486 RNP APCH SX GJA_24886, 8.5.3 - 1487 RNP_10 SX GJA_24888, |  |  |
| 8 .6 | This is an initial request for ETOPS long range operations for airplane type referenced in 8.1 ? YES NO Tick as applicable |  |  |
| 8.7 | 1. Aircraft capability to support 180 minutes reflected in : (Note :The operator has to submit the document reflecting the ETOPS capability to support 180 mins AFM/TCDS/etc) Operator response Refer to Attachment 8.3.1 & 8.7.1 & 8.7.1(2): TCDS No.: EASA.IM.A.169 Issue 6 & AFM Rev 10 & OIS-03 Rev 2 |  |  |
|  | 2. Previous reliability reports / Effectiveness of AMP (Note : Operator has to submit Reliability reports for the previous 12 mo) Operator response Refer to Attachment 8.7.2: 8.7.2 - SX-GJA Reliability data 100820 |  |  |
|  | 3. Engine Trend monitoring ETM performed in house YES □ NO □ ETM is contracted YES □ NO □ Provisions for early detection YES □ NO □ (Note : 1 Operator has to submit ETM reports for the previous 12 mo) 2. In case that ETM is contracted a copy of the contract must be attached to this conformance report) Operator response Refer to Attachment 8.7.3 : Note (1) 8.7.3 - FSR_Gainjet__BR725_21JUN2020_20JUL2020 Note (2) |  |  |
|  | 4. Oil consumption Procedure to monitor oil consumption (Note : Operator has to submit Oil consumption Procedure/ Reports for the previous 12 months Operator response Refer to Attachment 8.7.4: 8.7.4 - SX-GJA Oil Consumption Report 100820 |  |  |
|  | 5.Pre-departure check : Copy of Pre-departure check should be attached to this conformance doc (Note : A pre-departure check, additional to the pre-flight inspection required should be reflected in the Operator’s CAME to ensure that all maintenance actions are complete and all fluid levels are at prescribed levels for the flight duration) Operator response Refer to Attachment 8.7.5: AMP GJ-CA031A Issue.1 Rev.0 – Appendix A |  |  |
|  | 6 .Pre-departure check training Operator training should be attached to this conformance report Note:1) Pre-departure checks should be conducted and certified by an organization appropriately approved/accepted in accordance with PART-145 or by an appropriately trained flight crew member prior to an extended range flight . 2) Maintenance personnel should have an ETOPS familiarization training Operator response Refer to Attachment : 8.7.6 Note 1) Crew Limited Maintenance Training Records, Note 2) ETOPS Training for CAMO-P145 |  |  |
|  | 7 .Procedure to support critical tasks Part 145 In house : Yes Part 145 : Contracted Note 1) : 145.A.65 (b) 3 Safety and quality policy, maintenance procedures and quality system With regard to aircraft line and base maintenance, the organization shall establish procedures to minimize the risk of multiple errors and capture errors on critical systems, and to ensure that no person is required to carry out and inspect in relation to a maintenance task involving some element of disassembly/reassembly of several components of the same type fitted to more than one system on the same aircraft during a particular maintenance check. However, when only one person is available to carry out these tasks then the organisation’s work card or worksheet shall include an additional stage for re-inspection of the work by this person after completion of all the same tasks. Note 2: Refer to Contracted Part 145 Organization and how Note 1 is assured. Operator response Refer to Attachment 8.7.7.1/2/3/4 : 8.7.7.1 - AMP GJ-CA031A Issue.1 Rev.0 & 8.7.7.2 - CAME - Procedures for Contracted Maintenance, 8.7.7.3 - GNJ 145 - MOE ISS.2 REV.12, 8.7.7.4 - JAV - MOE Issue 17 Revision 00 |  |  |
|  | 8. Reporting malfunctions - Procedure should be established by the operator to report malfunctions related to ETOPS critical systems including all power plant events to the Airframe and Engine manufacturers as well as to the HCAA. (Refer to AMC 20-8) 2) These events should be evaluated by the operator in consultation with the HCAA. Operator response Refer to Attachment : 8.7.8 RELIABILITY REPORTING |  |  |
|  | 9. MEL Relative section of Aircraft MEL related to “extended range operations” should be provided Note: The MEL should take into account all items specified by the manufacturer relevant to ETOPS operations. Operator response Refer to Attachment 8.7.9: SX GJA MEL Issue 1 Rev 0_29 Jul 2020 |  |  |
|  | 10. Long Range Navigation System Manufacturer : Gulfstream PlaneView® II /Honeywell Primus Epic® Type installed : Planeview II Model: Planeview II TSO- N/A S/W version : Top level software P/N: EB7038683-00104 Operator response Refer to Attachment 8.7.10/1/2: Planeview Software P/N 6306 Aircraft logbook, 8.7.10.1 - AFM wip-01-01-34, 8.7.10.2 - Planeview II – 650 AOM 2B-01 thru 04 |  |  |
|  | 11. Communication systems: The operator has to refer to the Communication Systems installed on the aircraft. VHF : Yes (2) HF Yes (2) SATCOM Yes (1) ACARS TCAS II, 7.1 ACAS equivalent Other ADS-B 2, CPDLC (Note : Refer also to the number installed) Operator response Refer to Attachment 8.7.11.1/2.3 : 8.7.11.1 SX-GJA RADIO LICENCE -1364- 2020 HCAA, 8.7.11.2 Radio Equipment List CAMP 040820, 8.7.11.3 - ASC036B ADSB-2 6306 Aircraft logbook, 8.7.1.3 OIS-03 Rev 2 |  |  |
|  | 12. Operator copy of Aircraft tech Log Page : (Note : Operator has to submit a copy of a ATLS page) Operator response Refer to Attachment 8.7.12: ATLP 001, ATLP 002 |  |  |

# Operation Compliance Check List

| ITEM | COMPLIANT |  | REFFERENCE/REMARK |
| --- | --- | --- | --- |
| 9.1 Operation Manual | YES NO |  |  |
| Does the Operation Manual refers to non-ETOPS operation Note: Operator has to submit appropriate sections of O.M. | YES |  | OM Part A Section 8 Attachment 9.1 |
| 9.2 Flight Crew Training: |  |  |  |
| - Fuel management Verifying required fuel on board prior to departure and monitoring fuel on board en-route including calculation of fuel remaining. Procedures should provide for an independent cross-check of fuel quantity indicators. Confirmation that the fuel remaining is sufficient to satisfy the critical fuel reserves. | YES |  | OM Part A Section 8 Attachment 9.1 |
| - Procedures for single and multiple failures in flight that may give rise to go/no-go and diversion decisions | YES |  | OM PART A Section 8 Attachment 9.1 (AFM) & OM Operational Information Supplement 9.2/4 |
| - Policy and guidelines to aid the flight crew in the diversion decision making process and the need for constant awareness of the closest suitable alternate aerodrome in terms of time. | YES |  | OM Part A Chapter 8.5.10 Attachment 9.1 |
| - One-engine inoperative performance data Drift down procedures and one-engine inoperative service ceiling data. | YES |  | OM Part B6, SECTION 5 Att.9.2/5 AFM & OM Operational Information Supplement Attachment 9.1/2 |

| ITEM | COMPLIANT |  | REFFERANCE/REMARK |
| --- | --- | --- | --- |
| - Weather reports and flight requirements Weather reports and fl ight requirements: meteorological aerodrome reports (METARs) and aerodrome forecast (TAF) reports and obtaining in-fl ight weather updates on the en-route alternate (ERA), destination and destination alternate aerodromes. Consideration should also be given to forecast winds including the accuracy of the forecast compared to actual wind experienced during flight and meteorological conditions along the expected fl ight path at the OEI cruising altitude and throughout the approach and landing. | YES |  | OM Part A Section 8.1.1.8.2 Attachment 9.1 |
| - Pre-departure check Flight crew members who are responsible for the pre-departure check of an airplane should be fully trained and competent to do so. The training program required, which should be accepted by the HCAA, should cover all relevant maintenance actions with particular emphasis on checking required fluid levels. | YES |  | AMP G650 (GJ-CA031A), Appendix A Attachment 8.7.5 |
| - MEL | YES |  | MEL Issue 1 Rev 0 Attachment 8.7.9 |
| 9.3 Dispatch/Flight Planning Requirements: |  |  |  |
| - Fuel and oil supply Procedures established to dispatch an aeroplane for an extended range flight in order to verify that it carries sufficient fuel and oil to comply with the applicable operational requirements and any additional reserves determined in accordance with A. Critical fuel scenario B. Ice protection C. APU operation | YES |  | OM Part A 8.5.7.1/2/3 Attachment 9.1 |
| - Communication facilities The availability of communications facilities in order to allow reliable two-way voice communications between the airplane and the appropriate air traffic control unit at one-engine inoperative cruise altitudes. | YES |  | VHF #1 Voice VHF #2 Voice VHF #3 Voice/ Data & CPDLC Attachment 9.1/3 |

| ITEM | COMPLIANT |  | REFFERENCE/REMARK |
| --- | --- | --- | --- |
| - Aircraft Technical Log review to ensure proper MEL procedures, deferred items, and required maintenance checks completed. | YES |  | CAME - Chapter 1 par. 1.2.4. Attachment 8.7.9 |
| - En-route alternate aerodrome(s) Ensuring that en-route alternate aerodromes are available for the intended route, within 180 minutes based upon the one-engine inoperative cruise speed which is a speed within the certificated limits of the airplane | YES |  | OM Part A – SECTION 8 (8.5.4.1) Attachment 9.1 |
| One-engine inoperative cruise speed selected by the Operator and accepted by the Hellenic Civil Aviation Authority. | YES |  | 80/260 KCAS Attachment 9.2/3 |
| Procedures established confirming that, the weather conditions at en-route alternate aerodromes are at or above the applicable minima for the period of time during which the aerodrome(s) may be used | YES |  | OM Part A - SECTION 8, (8.5.8.3) Attachment 9.1 |
| 9.4 Operator Quality Compliance Monitoring and SMS regarding non-ETOPS |  |  |  |
| Refer how the QCM system monitors the non-ETOPS operations | YES |  | Management System Manual (MSM ) |

# APPLICATION PACKAGE

| Documentation to be submitted to the HCAA |  | Submitted? |  |
| --- | --- | --- | --- |
|  |  | Yes | No |
| 10.1 10.2 10.3 10.4 10.5 | This Compliance statement filled in which shows how the criteria of AMC1 CAT.OP.MPA.140(c), have been satisfied (Type Certificate Holder assistance may be required) . | X |  |
|  | The supporting documents as required by paragraphs 8 and 9 (as Attachments) | X |  |
|  | Flight crew ETOPS long range training program and syllabi for initial and recurrent training as per 9.2. | X |  |
|  | Operation manuals and checklists that include ETOPS long range operating practices and procedures (OM-A and/or OM-B and/or OM-D and/or OM C and /or AOM and/or FCOM and/or stand-alone Long Range manual). As per 9.1 9.3 Operator Quality Compliance Monitoring and SMS regarding non-ETOPS as per Par. 9.3 | X |  |
|  |  | X |  |

# OPERATOR'S  STATEMENT

| The undersigned certifies the above information to be correct and true and that airplane system installation, continuing airworthiness of systems, minimum equipment for dispatch, operating procedures and flight crew training comply with the requirements of 965/2012 - CAT.OP.MPA.140 (a)(2), 965/2012 - AMC1 CAT.OP.MPA.140(c) |  |  |
| --- | --- | --- |
| Name of Maintenance Manager: Jason Aivaliotis | Signature: | Date: |
| Name of Flight Operations Manager: Evangelos Lykoudis | Signature: | Date: |
| Name of Training Manager: George Dritsopoulos | Signature: | Date: |
| Name of Compliance Monitoring Manager Stavros Arampatzis | Signature: | Date: |

FOR OFFICIAL USE ONLY                                                               HCAA        APPROVAL

| Subject | Responsible | Date | Signature |
| --- | --- | --- | --- |
| 1. Compliance list for non-ETOPS long range operations airplanes application package checked for completeness | AWI/FOI |  |  |
| 2. Airworthiness Approval granted. | AWI |  |  |
| 3. Operational Approval granted (AOC, AOC Extract, or Letter of Authorisation). | FOI |  |  |
| 4. Non-ETOPS long range approval process administratively completed | AWI/FOI |  |  |
| non-ETOPS long range Approval Name: Date: Signature: |  |  |  |