|  | Approval | Cat II ## Cat III (check applicable box) |
| --- | --- | --- |
| FORM G5 b | ## OPERATIONAL DEMONSTRATION |  |

| Operator |  |
| --- | --- |

| INSPECTOR(S) | DATE |
| --- | --- |
| NON-COMPLIANCES /COMMENTS |  |

review of the documentation

Aircraft type which is new to the JAA into Category II or III service

| N° | Reference | Requirement | A | Operator’s answer (Refer to appropriate Manual/Section/ Paragraph) | Compliance criterion | C | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | AMC1.SPA.LVO.105 Operational Demonstration | (1) At least 30 approaches and landings must be accomplished in operations using the Category II/III systems installed in each aircraft type if the requested DH is 50 ft or higher. |  |  |  |  |  |
|  |  | If the DH is less than 50 ft, at least 100 approaches and landings will need to be accomplished unless otherwise approved by the Authority. |  |  |  |  |  |
|  |  | (2) If an operator has different variants of the same type of aircraft utilizing the same basic flight control and display systems, or different basic flight control and display systems on the same type of aircraft, the operator must show that the various variants have satisfactory performance, but the operator need not conduct a full operational demonstration for each variant. |  |  | The HCAA must define the number of required approaches and landings and the rate of successful approaches/landings |  |  |
|  |  | The Authority may also accept a reduction of the number of approach and landings based on credit given for the experience gained by another operator with an AOC issued in accordance with EC 965/2012 using the same aircraft type or variant and procedures. |  |  |  |  |  |
|  |  | (3) If the number of unsuccessful approaches exceeds 5 % of the total (e.g. unsatisfactory landings, system disconnects) the evaluation programme must be extended in steps of at least 10 approaches and landings until the overall failure rate does not exceed 5 %. |  |  | 2 An approach may be considered to be successful if: 2.1 From 500 feet to start of flare: a. Speed is maintained as specified in ACJ-AWO 231, paragraph [2 ‘Speed Control’]; and b. No relevant system failure occurs; and 2.2 From 300 feet to DH: a. No excess deviation occurs; and b. No centralised warning gives a go-around command (if installed). 3 An automatic landing may be considered to be successful if: a. No relevant system failure occurs; b. No flare failure occurs; c. No de-crab failure occurs (if installed); d. Longitudinal touchdown is beyond a point on the runway 60 metres after the threshold and before the end of the touchdown zone lighting (900 metres from the threshold); e. Lateral touchdown with the outboard landing gear is not outside the touchdown zone lighting edge; f. Sink rate is not excessive; g. Bank angle does not exceed a bank angle limit; and h. No rollout failure or deviation (if installed) occurs. |  |  |
|  |  | (c) Data Collection For Operational Demonstrations. Each applicant must develop a data collection method (e.g. a form to be used by the flight crew) to record approach and landing performance. |  |  |  |  |  |
|  |  | The resulting data and a summary of the demonstration data shall be made available to the Authority for evaluation. |  |  |  |  |  |

### Compliance criteria

| Comments N° | Comments |
| --- | --- |

### Comments

| Comments N° | Explanation |
| --- | --- |

A = applicability (Y : yes ; N no)

C = compliance (C : complied with ; NC : not complied with)